# Deep learning for regressive prediction of temperature based on raman spectrograms of different recycled PET concentrations.

Began by researching general knowledge of regression and classification algorithms. Found a deep learning network designed for the classification of raman spectrograms.

https://link.springer.com/article/10.1007/s00521-023-08700-z

Briefly tested the classification capabilities of the algorithm, it got 100% accuracy on the validation data, but didn’t test It on the test data. In order to do this some minor data processing was required of the CSV files.

Used chatgpt to modify the model for regression. The only change to the model itself was the output (just changed type of last dense layer).

The bulk of the conversion task was in the data processing and training of the model. Namely the loss function was changed (huber loss), encoding of outputs was removed and temperatures scaled.

The results were decent, I got I think an MAE of around 5.

I attempted to change some parameters to see if I could make anything better, but nothing improved it that much.

I spent a while researching into methods to improve the model. I attempted simply adding more layers also which resulted in a minor improvement. I experimented with filtering the data range based on the wave number in line with PETs absorption spectrum. The best results were from a range of 1100-1500 cm-1.

I also attempted feeding the rPET% into the model as contextual data to help the model learn. This produced varying results (using it as a second head in the model to aid the loss function, but its not an actual input into the model)

I considered trying to implement a GAN (generative adversarial network) after reading about how it produces artificial data to help itself learn. However this application was discovered to be invalid as the model would be just producing noise and not making changes relevant to the physics behind the raman scans. I also considered making tweaks to the data manually, to expand the dataset. This should be explored further, however applying those physics-relevant changes to the input data would still lack an equivalent physics accurate change in the temperature. So it wouldn’t be valid.

I also experimented with filtering the data using a Savitsky golay filter. The best results I got were when the filter had window length:200 and polyorder:40 on the input data, and wl: 50, plyo: 25 on the output data. With all of these factors combined, the MAE was reduced to 2.2, with a maximum prediction error of 4 degrees. However, after some diving into what the filter was doing, I found that it was actually completely flattening a lot of the data in the middle of the range. This could suggest that the algorithm learned better on the extreme ends of the data (It was after the 1100-1500 slice was applied), rather than the range that I had selected. Interestingly though, the filter applied on the testing side did not do this. So a model trained on almost fully flattened input data produced the most accurate preduction so far with unflattened* input data.

*test the second filter, not sure if it actually did flatten it or not. However this should be explored further.

Note 100 epochs (if earlystopping doesn’t kick in) produces the best results. I believe above 100 would take too long and give diminishing results. However should test.

I then used jordans PCA algorithm and applied the same process as I did with the rpet data, using it as a contextual aid to the model. However this made the results significantly worse, although only the second derivative method was used.

I then started the day with the objective of learning about how to evaluate a regression model in order to make some concrete changes that got me closer to an acceptable range of error. However I decided to just see if what I had currently would be able to produce any better results. So I spent the day creating a DOE setup. The parameters I was changing were the number of PCA components ( did 3 4 and 5 on the training and test data and exported them separately), the rpet and PCA loss weights, and the data slice range. I used doepy to create a full factor DOE table and then sampled 50 of them randomly. I edited the code so that I could call all the functions in one module, and set up a loop where each DOE component was input into the relevant functions, interpreted and used by the algorithm. I got about 100 tests after making different changes to the ranges. This didn’t produce any meaningful improvements. This was mainly due to the fact that with my DOE I didn’t allow for no PCA, which I believe was dragging the data down.

My next objective is to start again from scratch with this knowledge, to produce a modular and easy-to-evaluate setup where we can dive in to what produces t he most accurate results.

Project structure:

PCA: contains jordans PCA script

PCA_model: contains basic regression model using the PCA module

Ramannet_mod_reg: contains modified ramannet model, data processing etc

Codes: (under pca) contains original ramannet model modules]

Regression model: contains saved models for testing

DOE.txt contains parameters and consequent MAE of trained and tested model

Images -> ramannet_mod_reg2 restarting with a clean version of the model